NOM-002-SCT/2003

MEXICAN OFFICIAL JOURNAL	                                                              Wednesday, December 3, 2003

Mexican Official Standard NOM-002-SCT, List of hazardous substances and materias more usually transported

At the margin a seal with the National Crest, saying United Mexican States.- Secretariat of Communications and Transport. National Standardization Consultative Committees for Land, Air, Maritime and Ports Transportation.

MEXICAN OFFICIAL STANDARD NOM-002-SCT/2003, LIST OF HAZARDOUS SUBSTANCES AND MATERIALS MORE USUALLY TRANSPORTED.

AARON DYCHTER POLTOLAREK,  Under Secretary of Transport and President of the National Standardization Consultative Committees of Land transport and Air Transport, jointly with CESAR PATRICIO REYES ROEL,  Coordinator General of Ports and Merchant Marine and President of the National Standardization Consultative Committee of Maritime Transport and Ports, with foundation on articles 36, parts I, IV, VI, IX, XII, XIV, XVI, XVII, XVIII, XXV AND XXVII of the Organic Law of Federal Public Administration; 4th of the Federal Law of Administrative Procedures; 1st., 38 part II, 40 parts III, V, XIII, XVI and XVII; 41 and 43 of the Federal Law on Metrology and Standardization; 5th, part VI of the Law of Roads, Bridges and Federal Auto Transportation; 7th, part V, 39 and 63 of the Law of Navigation; 1st  and 6th, parts III and V of the Law of Civil Aviation; 1st and 17 of the Regulation for the Land Transport of Hazardous Materials and Wastes; 6th part XIII of the Interior Regulation of the Secretariat of Communications and Transport and other ordainments that may be applicable, and

CONSIDERING

That for the effects of safety and efficiency, it is necessary to establish for all different means of transportation, a uniformity in the Official Designation of the hazardous materials and substances, its corresponding UN number, the class of risk and the type of packaging and packages, as well as its special provisions.

That as a result of the commitments from the NAFTA between Mexico, the United States and Canada, on Chapter IX “Measures Concerning Standardization”, article 905 “Use of International Standards”, it is established that each part will use as a base for their own measures, as far as transport is concerned, the Recommendations of the United Nations Organization, Model Regulation, for the Transport of Dangerous Goods.

That having complied with the procedure established in the Federal Law on Metrology and Standardization and its Regulation, on February 7, 2003, it was published for general public comment the Project of the Mexican Official Standard PROY-NOM-002-SCT/2002, List of hazardous substances and materials more usually transported.

That with the prior authorization of the National Standardization Consultative Committees of Land Transport, Air Transport and Maritime Transport and Ports, it is now issued the Mexican Official Standard NOM-002-SCT/2003, List of hazardous substances and materials more usually transported.

Mexico, D.F. on the second day of the month of October of the year two thousand and three.- The President of the National Standardization Consultative Committees of Land Transport and Air Transport, Aarón Dychter Poltolarek.- Signature.- The President of the National Standardization Consultative Committee for Maritime Transport and Ports, César Patricio Reyes Roel.- Signature.

MEXICAN OFFICIAL STANDARD NOM-002-SCT/2003, LIST OF HAZARDOUS SUBSTANCES AND MATERIALS MORE USUALLY TRANSPORTED

PREFACE

In the elaboration of this Mexican Official Standard participated:

SECRETARIAT OF COMMUNICATIONS AND TRANSPORT

DIRECTORATE GENERAL OF FEDERAL AUTOTRANSPORTATION

DIRECTORATE GENERAL OF TARIFFS, RAILROAD AND MULTIMODAL TRANSPORTATION

DIRECTORATE GENERAL OF MERCHANT MARINE

DIRECTORATE GENERAL OF CIVIL AERONAUTICS

2.

DIRECTORATE GENERAL OF LEGAL AND JURIDICAL MATTERS

MEXICAN INSTITUTE OF TRANSPORTATION

SECRETARIAT OF THE INTERIOR

DIRECTORATE GENERAL OF CIVIL PROTECTION

NATIONAL CENTER OF DISASTER PREVENTION

SECRETARIAT OF THE ENVIRONMENT AND NATURAL RESOURCES

NATIONAL ECOLOGY INSTITUTE

DIRECTORATE GENERAL OF INTEGRAL DILIGENCE OF RISKY MATERIALS AND ACTIVITIES

SECRETARIAT OF NATIONAL DEFENSE

DIRECTORATE GENERAL OF THE FEDERAL REGISTRY OF FIREARMS AND EXPLOSIVES CONTROL

DIRECTORATE GENERAL OF MILITARY INDUSTRY

SECRETARIAT OF ENERGY

NATIONAL COMMISSION OF NUCLEAR SAFETY AND SAFEGUARDS

DIRECTORATE GENERAL OF LP GAS

SECRETARIAT OF LABOR AND SOCIAL PREVISION

DIRECTORATE GENERAL OF SAFETY AND HEALTH AT WORK

GOVERNMENT OF THE FEDERAL DISTRICT

DIRECTORATE GENERAL OF CIVIL PROTECTION

PETROLEOS MEXICANOS

NATIONAL CHAMBER OF TRANSFORMATIVE INDUSTRY

NATIONAL CHAMBER OF CARGO AUTO TRANSPORT

NATIONAL ASSOCIATION OF THE CHEMICAL INDUSTRY

MEXICAN ASSOCIATION OF NON DESTRUCTIVE TEST COMPANIES

MEXICAN ASSOCIATION OF PAINTS AND DIES MANUFACTURERS

NATIONAL ASSOCIATION OF AROMATIC PRODUCTS MANUFACTURERS

ASSOCIATION OF CARGO CARRIERS OF THE CENTRAL ZONE OF THE STATE OF VERACRUZ

NACIONAL DE CARROCERIAS, S.A. DE C.V.

GRUPO INTERMEX

DUPONT DE MEXICO, S.A. DE C.V.

GRUPO INDUSTRIAL RESISTOL

DESC CORPORATIVO, S.A. DE C.V.

GRUPO TMM, S.A.

INDEX

- Objective

- Field of application

- References

3.

- Definitions

- Classification and designation of the hazardous substances and materials

- Tables Annexes to the Standard

- Bibliography

- Harmonization with international standards and lineaments

- Enforcement

- Compliance

- Sanctions

- Validity

- Transitory

- Objective

This Mexican Official Standard has as an objective to identify and classify the hazardous substances and materials more usually transported, according to its class, risk division, subsidiary risk, number assigned by the Organization of the United Nations, as well as the special provisions to which its transportation must be subject to as well as the method of package and packaging.

- Field of Application

This Mexican Official Standard is obligatory, within the sphere of their responsibilities for shippers, carriers and receivers of the hazardous substances and materials, transported through the general means of communication by land, maritime and air.

- References

For the correct application of this Standard, it is necessary to consult the following Mexican Official Standards, or those superseding them:

NOM-052-ECOL/93   ESTABLISHING THE CHARACTERISTICS OF HAZARDOUS WASTE, A LIST OF THEM AND THE LIMITS THAT MAKES A HAZARDOUS WASTE DANGEROUS, DUE TO ITS TOXICITY TO THE ENVIRONMENT.

NOM-053-ECOL/93	      ESTABLISHING THE TEST PROCEDURES TO PERFORM THE EXTRACTION TEST FOR THE ELEMENTS THJAT MAKES A WASTE OR RESIDUE DANGEROUS FOR ITS TOXICITY TO THE ENVIRONMENT.

NOM-054-ECOL/93  ESTABLISHING THE PROCEDURE TO DETERMINE THE INCOMPATIBILITY BETWEEN TWO OR MORE RESIDUES, CONSIDERED AS DANGEROUS BY NOM-CRP-001-ECOL.

- Definitions

Hazardous Substance.-  Every element, combination, material, or mixture of them, that independently of its physical state, represents a potential risk for health, environment, safety of the users and property of third party ones; also, under this definition are considered the biological agents which are cause of diseases.

Hazardous material.- Those dangerous substances, their residues, their packaging, packages, and all other component that constitute the cargo that will be transported by the units.

GE (PG).- Packaging Group.

Shipper.- A person or company that in its own name or of a third party one, hires the services of transportation of hazardous materials.

Carrier.- Auto transporter, railroad, maritime or aerial company.

Consignee.-  The person or company receiving hazardous materials.

The terms and definitions not contained in this point and that the Secretariat of the Environment and Natural Resources, this Secretariat of communications and transport or the corresponding agencies to which this apply, will be understood as defined in the terms provided by the laws, regulations, in its case, the definitions originated in international instruments, ratified by the Mexican Government.

4.

5.- Official transportation designation and classification of the hazardous substances and materials.

For the identification of the hazardous substances and materials by its class of risk or division, UN number assigned by the United Nations Organization, inherent subsidiary risks, as well as special provisions to which its transportation must be subject to, the group and method of packaging and packages that must be utilized, will be performed according to what is established on Tables 1 and 2 of this Mexican Official Standard.

In the case that the official designation for the transport of a hazardous substance or material is not expressly considered on Tables 1 and 2 of this Standard, the shipper will establish the classification, by taking the generic designations listed on Table 4 of this Standard, considering for such action the predominant dangerous characteristics of the substance or material in the case, for which and if so considered convenient, can be remitted to the MSDS, or through an approved and accredited testing laboratory.

  - General provisions

    - Tables 1 and 2 of this Mexican Official Standard, are not exhaustive, they include only the most usually transported hazardous substances and materials. The tables will continue to be completed with the end of integrate as far as possible, all the hazardous substances and materials that due to its commercial importance, are transported more frequently. This will be achieved through a modification to the Standard, published on the Federation Official Journal and according to the modifications and actualizations of the latest edition of the Model Regulation of the United Nations, Recommendations on the Transport of Dangerous Goods.

5.1.2The substance or material shown expressly by its name on the Lists of Hazardous Substances and Materials, will be transported according to the provisions established for that substance or material. To do the transportation of hazardous substances or materials that are not expressly mentioned in the Lists of Hazardous Substances and Materials, it must be used a “generic” term that contains the “not otherwise specified” (N.O.S.). The subject substance or material, can only be transported when their intrinsic dangerous physical-chemical properties have been determined, after which, it will be classified according to the definitions and criteria that describes it more adequately. The Shipper himself will proceed to classify his hazardous substances and materials, or else, through the results of tests performed and guaranteed by an accredited laboratory, that will attest that such substances or materials do not satisfy the parameters to be considered as such. Once the class to which the substance or material belongs to, compliance of all the requisites for the shipping and transporting which are established on the proper regulation must be complied with.

    - Substances or materials that are presumed to have explosive characteristics, must be considered primordially to include them under class 1. Some collective epigraphs may be of the generic type or “ Not Otherwise Specified”, as far as its safety is always guaranteed both excluding the transport of normal conditions to the substances or materials extremely dangerous, as well as having in consideration all the subsidiary risks that may be present on some substance or material.

    - The lists of hazardous substances and materials, do not include substances and materials for which transportation is forbidden or controlled, in certain means of transportation, partly because it would be impossible to have an exhaustive list of them. However, the fact that a substance or material is not precisely noted on the Lists of Hazardous Materials which include the substances and materials more usually transported, should not be considered that its transportation can be done without restrictions, since the instability inherent to certain substances and materials may represent certain risks like explosion, polymerization with strong heat emission or release of toxic gases, which in their majority may be avoided through the utilization of proper packages and packaging or else, through procedures of dilution, stabilization, addition of some inhibitor, refrigeration or any other special caution provisions. In these cases the procedure has to be in accordance with point 5.1.2 of this Standard.

    - When caution measures are prescribed on the List of Hazardous Substances and Materials for a determined substance or material (for example, that they are “stabilized” or “contain a determined percentage (%) of water or phlegmatizer”), that substance or material can not be normally transported, if such measures have not been taken, unless the material appears indicated in another section (for instance, on class 1), without any indication relative to caution measures or with the indication of different measures.

  - Official transport designation

    - The official transport designation is the part of the denomination that better describes the hazardous substances and materials and that appears in upper case letters on the List of Hazardous Substances and Materials, in some cases with digits, Greek letters or the prefixes “sec”, “terc”, m, n, o, p, that are an integral part of the designation. Some times between brackets another official transport designation is prescribed after the principal designation, for example: ETHANOL (ETHYLIC ALCOHOL).

    - The parts of a denomination appearing in lower case letters, are not considered elements of the official designation of transport, excepting in the case of indications as “sec-“, “terc-“, “m-“, “n-“, “o-“ and “p-“.

    - If there are conjunctions like “and” or “or” in lower case letters or if in some elements of the nameare separated by comas, it is not necessary to indicate that name wholly in the shipping document or in the markings of the packages and packaging. This is the case, particularly, when a combination of several different denominations that is shown with a single UN number.

5.

5.2.4   The following examples show how the Official designation for transport must be chosen in such cases:

- UN 1057 LIGHTERS or LIGHTER REFILLS- It will be considered as Official designation for transport, the most appropriated of the two following designations:

LIGHTERS

LIGHTER REFILLS

- UN 3207 ORGANOMETALLIC COMPOUND or COMPOUND SOLUTION or COMPOUND DISPERSION, WATER REACTIVE, FLAMMABLE, N.O.S. As an official designation for transport, the best of the following combinations can be utilized:

ORGANOMETALLIC COMPOUND, WATER RECTING, FLAMMABLE, N.O.S.

ORGANOMETALLIC SOLUTION, WATER REACTING, FLAMMABLE, N.O.S.

ORGANOMETALLIC DISPERSION, WATER REACTING, FLAMMABLE, N.O.S.

As a complement for each one of the generic denominations (N.O.S.), to the official designation for transport will be added the technical name of the material, the generic denominations with respect of which is considered necessary to add that data, have assigned on column 6 of table 2 of this Standard, Special Provision 274, except that a national law or international agreement prohibits to divulge it.

5.2.5  When an official designation for transport is assigned to a hazardous substance or material as a “N.O.S.” generic denomination, to which have also been assigned Special Provision 274, it must be added (immediately after the “N.O.S.” generic denomination) in brackets the technical name or approved chemical name or another name that is of common use in manuals, periodic publications and scientific or technical texts. Commercial brands or names should not be utilized for this end. In the case of pesticides, only the common name approved by the International Standards Organization (ISO) should be utilized, or other names included in the Recommended Classification of Pesticides by Hazard and Guidelines to Classification) of the World Health Organization (WHO) or in the Exclusive Pesticide Catalog of the Intersecretariat Commission for the Control and Use of Pesticides, Fertilizers and Toxic Substances (CICOPLAFEST), or else the name or names of the active ingredients of the substances.

Adequate expressions  can also be used, such as: “contains”, or “containing” or other names such as “mixture”,“solution”, etc., as well as the percentage of the technical component, for example: UN 1993 FLAMMABLE         LIQUID N.O.S. (CONTAINS XYLENE AND BENZENE), 3, PG II.

    - When a mixture of hazardous materials is described with one of the “generic denominations” or “N.O.S.”, to

which have been also assigned Special Provision 274, on column 6 of table 2 of the Lists of Hazardous Substances and Materials, it will be indicated in brackets the two predominant risk components of the mixture. This is not applicable if any of the components of predominant risk is a controlled substance according to the national laws or an international agreement that prohibits its divulging it. If a mixture of hazardous materials has a compound that carries a label of subsidiary risk, this compound must be shown as one of the two technical names appearing in brackets.

The following examples show how to choose the official designation for transport, along with the technical name, in the case of materials carrying the “N.O.S.” indication.

UN 2003		METAL ALKYLS, N.O.S. (trimethylgalium)

UN 2902		PESTICIDE, LIQUID, TOXIC, N.O.S. (drazoxolon).

    - The official designation for transport may be used in singular or plural, according to the case. However, if portions of it contain terms that delimitate its sense, the order of these in the documents or the markings of the packages is facultative, for example: “ PHENYLMERCURIC, COMPOUND”, can also be shown as “ COMPOUND, PHENYLMERCURIC”.

    - For materials of class 1 commercial or military names containing the official designation for transport can be used, complemented with a descriptive text.

    - Unless it already appears on the Lists of Hazardous Substances and Materials in upper case letters with its appropriate name and due to the various physical states of its diverse isomers, that may be a liquid or a solid (for example: DINITROTOLUENES, LIQUID; DINITROTOLUENES, SOLID), to the official designation of transport will be added, the qualifications “LIQUID” or “SOLID” according to the case, or “MOLTEN”, when a substance that is solid is presented for its transportation in a molten state (for example: ALKYLPHENOLS, SOLID, N.O.S., MOLTEN)

Save for the substances of spontaneous reaction and organic peroxides and unless the name is already shown in upper case letters on column 2 of the lists of hazardous materials, the word “STABILIZED” will be added, as part of the official designation of transport of a substance that without stabilization, it would not be feasible to be transported since it may react in a dangerous manner under normal transportation conditions. Example: “ ORGANIC TOXIC LIQUID, N.O.S., STABILIZED”.

6.

  - Mixtures and solutions containing a hazardous substance

    - Every mixture or solution that contains a hazardous substance expressly mentioned on the Lists of Hazardous Substances and Materials and one or several substances that are not included on the Lists, must be treated according to the requirements established for the hazardous substances, indicating what package or packaging is appropriate to the physical state of the mixture or solution, unless:

  - The mixture or solution is expressly listed in this Standard;

  - On the specific section designated on this Standard that indicates explicitly that the denomination refers only to  the substance or material in its pure state;

  - The class of risk, the physical state or the packing group of the solution or the mixture are different to those of the hazardous substance or material;

  - There is a significant change on the measures to be adopted on emergency situations.

    - For the solutions and mixtures that are classified according to the provisions relative to the hazardous substance, on the official designation of transport will be added, according to the case, the word “SOLUTION” or the word “MIXTURE”, for example: “ACETONE IN SOLUTION”. Also the concentration of the solution or mixture must be indicated, for example: “ACETONE, SOLUTION AT 75%”.

    - Mixtures or solutions containing one or more substances expressly mentioned on the Lists of Hazardous Substances and Materials or classified under a N.O.S. entry and one or various substances not subject to the Regulations for the transport of hazardous substances and materials, will be exempt of the application of this last one if the risk characteristics of the mixture or solution are such that does not satisfy criteria of any class or division (including criteria of human experience).

  - Preponderance of the risk characteristics

- Table 5 must be utilized as a guide to determine the class in which a substance or material should be included, a mixture or solution that presents more than one risk, when these do not appear on the list of hazardous substances and materials of table 2.

- For substances and materials that have multiple risks and that are not listed specifically by its official designation for the transport on table 2, the most rigorous group of packaging indicated for the respective risk of the substances and materials, will have priority over the other packaging groups.

- The priority of the risk characteristics of the following materials or substances that have not been taken in consideration within table 5 of risk preponderance, since its primary characteristics will always have priority.

- Substances and objects of Class 1

- Gases of Class 2

- Spontaneous reaction and similar substances and desensitized explosives of Division 4.1

- Pyrophoric  Substances of Division 4.2

- Substances of Division 5.2

- Substances of Division 6.1 toxicity by inhalation with a packing group I, *

- Substances of Division 6.2

- Materials of Class 7

- Excepting the substances or mixtures that comply with the criteria of Class 8, having a toxicity by inhalation or powder or vapors (CL50 “lethal concentration 50”), in the rank of packing group I, but for toxicity through oral ingestion or skin contact, only on the packing group III rank or less, which should be assigned to Class 8.

    - In all shipping documents, must be always indicated, the most specific applicable denomination to the hazardous substance or material.

7.

6.	Tables annexed to the Standard

Table 1:	List of Hazardous Substances and Materials more Usually Transported, by alphabetical order.

Table 2:	List of Hazardous Substances and Materials more Usually Transported, by numerical order.

Table 3:	Special Provisions Relative to Determined Substances and Materials.

Table 4:	List of Official Generic Denomination for Transportation or corresponding to Substances and Materials Not Otherwise Specified.

Table 5: 	Order of Preponderance of the Risk Characteristics ( Risk Class and Packing Group)

Table 6:	List of Substances of Spontaneous Reaction.

  - General Indications for Consultation of table 1

List of Hazardous Substances and Materials more Usually Transported, by alphabetical order, and is interpreted as follows:

a) This table is not a general index of provisions, it is a list in alphabetical order of the substances and materials that are listed in numerical order on table 2 and designed with a format of three columns that indicate the following:

First Column, shows the name of the substance or material

Second Column, shows the risk class and division to which the substance or material belongs to.

Third Column, shows the assigned number by the United Nations Organization, for the substance or material.

b)  	To establish the alphabetical order, although being part of the official designation of transport, it was not taken in consideration the following elements: numbers; names of the Greek letters, abbreviations “sec” and “terc” and the letters “N” (nitrogen); “n” (normal), “o” (ortho), “m” (meta [goal]), “p” (para [for]) and N.E.P. (N.O.S.).

- The name of a substance or material in upper case letters constitutes an official designation of transport.

- The name of a substance or material in upper case letters followed by the expression “see or look for” constitutes an optional official designation for transport or else, it is part of an official designation of transport.

- A denomination in lower case letters followed by the expression “see or look for”, is not an official designation of transport, but a synonym.

- In a denomination containing a part expressed in upper case letters and another in lower case letters, it will be understood that the latter is not a part of the official designation of transport.

- The official designation of the substance or material may be used in singular or plural, as it may be more appropriate for the purposes of the shipping documents and the marking of the package and packaging.

  - General Consultation Indications for Table 2

Table 2 List of Hazardous Substances and Materials more Usually Transported, by numerical order, is divided in 11 columns which indicate the following:

_______________________________________________________________________________________________

Column 1:	“UN N°”: contains the serial number assigned to the substance or materials in the United Nations system.

_______________________________________________________________________________________________

Column 2:	“Name and Description”: official designation of transport is indicated, in upper case letters, sometimes followed by a descriptive text shown in lower case lettering. Official designation of transport may be given in plural when there are isomers of the same classification. Hydrates of organic substances may be included, according to the case, under the official designation for transport of the anhydrous substance.

_______________________________________________________________________________________________

Column 3:	“Class or Division”: points out the class or division and in the case of class 1, the compatibility group assigned to the material or substance according to the classification system described in section 5.

_______________________________________________________________________________________________

Column 4:	“Subsidiary Risk” this column contains the number of class or division of the important subsidiary risks that have been determined, applying the classification system described in section 5.

_______________________________________________________________________________________________

8.

______________________________________________________________________________________________

Column 5:	“UN packing/packaging group”: Points out  the number of the group of packing and packaging of the United Nations (I, II or III) assigned to the material or substance. If more than one group is indicated for the product in question, the group of package or packaging of the substance or preparation to be transported, will be determined according to its properties, applying the classification criteria established on section 5.

_______________________________________________________________________________________________

Column 6:	“Special Provisions: In it, there is a number that takes you to the special provisions of table 3, applicable to the material or substance. Special Dispositions are applied to every authorized packing group for a determined substance or material, except that the text indicates clearly another thing.

_______________________________________________________________________________________________

Column 7:	“Limited Quantities”: in this column it is shown the maximum quantity by package and interior packing authorized for the transport of the substance or material in question, according to the provisions of the Mexican Official Standard NOM-011-SCT 2, that relates to limited quantities. The word “None” in this column , means that it is not permitted to transport the substance or materials under the Standard of limited quantities.

_______________________________________________________________________________________________

Column 8:	“Instructions for packing/packaging” This column contains alphanumerical codes that takes to the  corresponding special provisions of packing and packaging of the corresponding Standard. In the special instructions of packing and packaging, the special provisions for packing and packaging are indicated, including the intermediate bulk containers (IBC’s) as well as the large packages and packaging that may be utilized for the transport of hazardous substances and materials.

A code that includes the letter “P” makes reference to the instructions of packages and packaging applicable to packages and packaging described in the proper Standard for organic peroxides.

A code including the letters “IBC” makes reference to the instructions of package and packaging of applicable to the usage of IBC’s.

A code that includes the letters “LP” makes reference to the instructions of packaging for the usage of large packages and packaging.

When no specific code is signaled, it will be considered that the substance or material is not authorized for the type of package and packaging that may be used in the frame of the instructions for packages and packaging carrying that code.

If in the column appears the letters N/R, it means that it is not necessary to pack or use packages for the substance or material in question.

_______________________________________________________________________________________________

Column 9:	“Special Provisions for Packages and Packaging”: this column contains alphanumerical keys that takes you to the corresponding special provisions of packages and packaging of the appropriate Standard. In the special instructions of package and packaging are indicated the special provisions of package and packaging (including the IBC’s and large volume packages).

A special provision for package and packaging that includes the letters “PP”, indicates that there is a special provision applicable to the use of the instructions for package and packaging that carries the code E in the corresponding Standard.

A special provision including the letter “B”, refers to a special provision for package and packaging  applicable to the use of the instructions for package and packaging that carry the code IBC within the corresponding Standard

A special provision including the letter “L”, makes reference to the special provisions of package and packaging that carry the code “LP” in the corresponding Standard.

_______________________________________________________________________________________________

Column 10:	“ Instructions for portable tanks”:  this column shows a number preceded of the letter “C” that takes to the corresponding instruction of the appropriate Standard, that prescribes the type or types of tanks demanded for the transport of the substance or materials in portable tanks.

_______________________________________________________________________________________________

Column 11:	“Special provisions for portable tanks”: In this column a number preceded by the letters “CP” is indicated, that takes to the special provisions that are applied to the transport of the substance or material in portable tanks

_______________________________________________________________________________________________

9.

    - Abbreviations and symbols

On the Hazardous Materials list the following abbreviations or symbols are utilized with the meanings indicated below:

_______________________________________________________________________________________________

Abbreviation           Column			Meaning

_______________________________________________________________________________________________

N.O.S.	         2			Not Otherwise Specified.

_______________________________________________________________________________________________

†	                           2			Item in which it is possible to consult a more detailed explanation for

its application.

_______________________________________________________________________________________________

  - General Consultation Indications of table 3

Special Provisions Relative to Hazardous Substances and Materials

- Special provisions relative to determined substances or materials, are indicated on table 3. These provisions are identified by Arabic digits and are not necessarily found in numerical order.

  - General Consultation Indications of table 4

List of Official Designations for Transport, Generic or Corresponding to Groups of Substances and Materials not Specified in Other Place.

- Substances and materials not appearing with their proper name in the provisions of table 2, must be classified according to what is indicated on part 5 of this Standard, determining as the official designation for transport, that, among those named on that listing, suits best the identification of the substance or material to deal with, thus, the names on Table 2 that describe the substance or material, must be used as the shipping name of the hazardous substance or material in question.

- The list, besides the main generic items given on Table 2, considers all the corresponding denominations to groups of substances “not otherwise specified”.

    - In this list, the generic names or N.O.S. are grouped according to its risk class or division. The names have been placed into three groups as follows:

- Specific denominations, covering a group of substances or materials of determined chemical or technical characteristics.

- Denominations for pesticides of Class 3 and Division 6.1, and

- General denominations, corresponding to groups of substances or materials that have one or more dangerous properties in general.

  - General  Consultation Indications of table 5

Table 5, Order of Preponderance of the Risk Characteristics, must be utilized to determine the class in which a substance or material should be included, a mixture or solution that presents more than one risk, when such substance or material, mix or solution presenting more than one risk, when such substance or material, mix or solution is not classified on tables 1 and 2 of this Standard.

  - General Consultation Indications for table 6

In this table, the classification by reference has been determined  for those substances that are technically pure, save for the cases in which a concentration less than 100% is indicated, or when the concentration is different, the substances can then be classified in a different manner.

- Bibliography

  - Federal Law on Metrology and Standardization and its Regulation

  - Recommendations on the Transport of Dangerous Goods, Model Regulation, issued by the United Nations Organization, Twelfth Revised Edition, New York and Geneva, 2001.

  - International Maritime Code for Dangerous Goods (IMDG9; Amendment 29-96

  - Annex 18 to the Chicago Agreement on International Civil Aviation DOC 9284-AN/905 of the Technical Instructions for the Safe Transport of Dangerous Goods by Air ICAOI (1997-1998)

- Harmonization with international standards and regulations

This Official Mexican Standard is equivalent to:

  - Recommendations on the Transport of Dangerous Goods, Model Regulation, issued by the United Nations Organization, Twelfth Revised Edition, New York and Geneva, 2001.

  - International Maritime Code for Dangerous Goods (IMDG) Amendment 29-96

  - Annex 18 to the Chicago Agreement on International Civil Aviation DOC. 9284-AN/905 of the Technical Instructions for the Safe Transport of Dangerous Goods by Air ICAO (1997-1998)

10.

- Supervision

The Secretariat of Communications and Transport, through the Directorates Generals with authority, is the competent Authority to watch the compliance of this Official Mexican Standard.

- Compliance

This Standard is obligatory in the General Means of Communication for the Transport of Hazardous Substances and Materials, with foundation in the Civil Aviation Law and its Regulation; Law of Navigation and its Regulation; Law of Ports and its Regulation; Regulation for the Transport of Hazardous Materials and Wastes and any other international documents signed by our country for the land, air and sea transportation.

11.	Sanctions

Non compliance of what is established in this Mexican Official Standard will be sanctioned by this Secretariat, according to what is established in the Law of Civil Aviation and its Regulation; Law of Navigation and its Regulation; Law of Ports and its Regulation; Regulation for the Transport of Hazardous Materials and Wastes and all other legal ordainments that may result applicable, without prejudice of those imposed by other agencies of the Federal Executive branch of the Government while exercising their attributions or of the resulting civil or criminal responsibility.

12.	Validity

This Mexican Official Standard will enter in vigor 60 days after its publication in the Federation Official Journal.

13.	Transitory

FIRST. This Mexican Official Standard, substitutes NOM-002-SCT2/1994, List of Hazardous substances and materials more usually transported, published in the Federation Official Journal dated October 30, 1995.

SECOND. Columns from 8 to 11 of table 2 of this Standard are illustrative, as far as they are published and actualized the appropriate Mexican Official Standards. They may also be valid for the international trade.