Translation of NOM-007

_______________________________________________________________

SECRETARIAT OF COMMUNICATIONS AND TRANSPORT

Mexican Official Standard NOM-007-SCT2/2002, Markings of packages and packaging destined to the transport of hazardous substances and wastes.

____________________________________________________________________________

At the margin a seal with the National Crest that says: United Mexican States.- Secretariat of Communications and Transport.

MEXICAN OFFICIAL STANDARD NOM-007-SCT2/2002, MARKINGS OF PACKAGES AND PACKAGINGS DESTINED TO THE TRANSPORT OF HAZARDOUS MATERIALS AND WASTES.

AARON DYCHTER POLTOLAREK, Under Secretary of Transport and President of the National Consultative Committee for Standardization of the Land Transport, with foundation on articles 36 fraction I of the Organic Law of the Federal Public Administration; 1st., 38 fraction II, 40 fractions I, V and XVI, 41, 43 and 51 of the Federal Law on Metrology and Standardization; 4th. Of the Federal Law of Administrative Procedures; 5th. Fraction VI of the Federal Law of Roads, Bridges and Auto Transport; 34 of the Regulation of the Federal Law on Metrology and Standardization; 1st., 19 and 32 of the Regulation for the Land Transport of Hazardous Materials and Wastes and 6th. Fraction XIII of the Interior Regulation of the Secretariat of Communications and Transport and any other legal ordainment that may result applicable, and

CONSIDERING

That the packages and packaging utilized to transport hazardous substances and wastes require bearing the corresponding marks that guarantees its integrity, by going through and passing the laboratory testing procedures.

That having complied with the procedure established in the Federal Law on Metrology and Standardization and its Regulation for the issuing of Mexican Official Standards, the Under Secretary of Transport and President of the National Consultative Committee for the Standardization of the Land Transport, on August 14, 2002, ordered the publication in the Federation Official Journal of the Project for the Mexican Official Standard PROY-NOM-007-SCT2/2002, Marking of the Packages and Packaging destined to the Transport of Hazardous Substances and Wastes, which establishes the characteristics and specifications that must be complied with for the marking of the packages and packaging, destined to the transport of hazardous materials, substances and wastes, in order to submit it to the public opinion.

That during the period of 60 natural days, counted as of the date of publication of the Project for the Mexican Official Standard, the Manifest of Regulatory Impact and the analysis that served as a base for its elaboration, referred on article 45 of the Federal Law on Metrology and Standardization and 32 of its Regulation, were at the disposal of the public in general for its consultation.

That the Decree of the Promulgation of the Agreement 170, on article 7th. “Labeling and Marking”, points out on point 1 that: “all chemical products must carry a mark that allows its identification. Also on point 3.1 points out that for the marking or labeling of chemical products, these demands must be established by the competent authorities or by an approved and recognized Organism by a Competent Authority in conformity with the National or International Standards”

That this Mexican Official Standard, recognizes the recommendations issued by the Committee of Experts on the Transport of Dangerous Goods of the United Nations Organization in December of 1999.

That the Federal Law on Metrology and Standardization, on article 51, fourth paragraph, establishes the obligation of revising the Mexican Official Standards every five years. In this sense, once the revision and the corresponding notification are performed and due to the fact that it is indispensable to application of the general precepts established within the Standard, its modification was determined, with the purpose of  actualizing it to the eleventh edition of the United Nations, without essentially modifying its contents nor its identification codes.

That  the modification of this Standard is necessary, in view that the international guidelines with which it is actually harmonized and served as a base for its elaboration, were actualized, also it is fundamental to incorporate within it the general criteria in matters of Evaluation of Conformity, to the effect of granting security and legality to the manufacturers, consumers and users of the packages and packaging utilized for the transportation of hazardous materials, substances and wastes.

NOM-007-SCT2/2002     Page 2

That given the importance of the utilization of adequate packages and packaging, destined to contain these materials during their transportation, it is necessary to be performed under conditions that guarantees the security in the usage of the transportation services and that this represents a minimal risk for the people and the environment.

For all the above, I am issuing the following:

Sincerely,

Mexico City, April 7, 2003.- The Under Secretary of Transport and President of the consultative Committee for the Standardization of the Land Transport, Aarón Dychter Poltolarek.- Signed

MEXICAN OFFICIAL STANDARD NOM-007-SCT2/2002 MARKINGS OF PACKAGES AND PACKAGINGS DESTINED TO THE TRANSPORT OF HAZARDOUS MATERIALS AND WASTES

PREFACE

In the elaboration of this Mexican Official Standard participated the following:

SECRETARIAT OF COMMUNICATIONS AND TRANSPORT

DIRECTORATE GENERAL OF FEDERAL AUTO TRANSPORT

DIRECTORATE GENERAL OF TARIFFS, RAILROAD AND MULTIMODAL TRANSPORTATION

DIRECTORATE GENERAL OF MERCHANT MARINE

DIRECTORATE GENERAL OF LEGAL COUNSELING

MEXICAN INSTITUTE OF TRANSPORTATION

SECRETARIAT OF THE INTERIOR

DIRECTORATE GENERAL OF CIVIL PROTECTION

NATIONAL CENTER OF PREVENTION OF DISASTERS

SECRETARIAT OF PUBLIC SAFETY

FEDERAL PREVENTIVE POLICE

SECRETARIAT OF THE ENVIRONMENT AND NATURAL RESOURCES

NATIONAL INSTITUTE OF ECOLOGY

DIRECTORATE GENERAL OF INTEGRAL CONTAMINANTS HANDLING

SECRETARIAT OF NATIONAL DEFENSE

DIRECTORATE GENERAL OF FEDERAL REGISTRY OF FIREARMS AND EXPLOSIVES CONTROL

DIRECTORATE GENERAL OF MILITARY INDUSTRY

SECRETARIAT OF ENERGY

NATIONAL COMMISSION OF NUCLEAR SAFETY AND SAFEGUARDS

DIRECTORATE GENERAL FOR L.P. GAS

NOM-007-SCT2/2002    Page 3

SECRETARIAT OF HEALTH

DIRECTORATE GENERAL OF ENVIRONMENTAL HEALTH

SECRETARIAT OF LABOR AND SOCIAL PREVISION

DIRECTORATE GENERAL OF  SECURITY AND HEALTH AT WORK

GOVERNMENT OF THE FEDERAL DISTRICT

DIRECTORATE GENERAL OF CIVIL PROTECTION

PETROLEOS MEXICANOS

INDUSTRIAL SECURITY, ENVIRONMENT PROTECTION AND ENERGY SAVINGS AUDIT OFFICE.

NATIONAL CHAMBER OF THE TRANSFORMATION INDUSTRY (CANACINTRA)

NATIONAL CHAMBER OF CARGO AUTO TRANSPORT (CANACAR)

NATIONAL ASSOCIATION OF THE CHENICAL INDUSTRY, A.C.(ANIQ)

NATIONAL ASSOCIATION OF MANUFACTURERS OF PAINTS AND DYES A.C. (ANAFAPYT)

NATIONAL ASSOCIATIONOF MANUFACTURERS OF AROMATIC PRODUCTS A.C. (ANFPA)

MEXICAN ASSOCIATION OF NON DESTRUCTIVE TESTS COMPANIES, A.C. (AMEPND)

CARGO CARRIERS ASSOCIATION OF CENTRAL STATE OF VERACRUZ, A.C. (ATCCEVAC)

GRUPO INTERMEX

DESC, S.A. DE C.V.

DUPONT, S.A. DE C.V.

INDEX

- Objective

- Field of application

- References

- Definitions

- Specifications for the marking of packages and packaging

- Bibliography

- Harmonization with Standards and international guidelines

- Compliance

- Inspection

- Evaluation of Conformity

- Sanctions

- Validity

- Transitory

  - Objective

This Mexican Official Standard has as an objective to establish the characteristics and specifications that must be complied with, for the marking of the packages and packaging destined to the transportation of hazardous substances, materials and wastes.

  - Field of Application

This Mexican Official Standard within the sphere of its responsibilities, is of obligatory compliance for the shippers and consignees of the materials, substances and wastes, and of the manufacturers responsible for the construction and reconstruction of the packages and packaging that are utilized for its transportation.

The packages and packaging that are delivered for its use in transportation must comply  with the provisions established in this Standard.

This Standard is not applicable for:

- Packages and packaging destined to have as contents, materials of class 7, radioactive, or its residues, which will be subject to the standards issued by the Secretariat of Energy, through the National Commission of Nuclear Safety and Safeguards.

- Packages and packaging for the transportation of refrigerated compressed gases, liquefied or pressure dissolved, class 2, and

NOM-007-SCT2/2002     Page   4

c) 	Packages and packaging whose net weight exceeds 400 Kg. or whose capacity exceeds 450 liters.

  - References

For the correct application of this Standard, it is necessary to consult the following Mexican Official Standards or those substituting them:

NOM-002-SCT	List of Hazardous Substances and Materials more Usually Transported.

NOM-024-SCT2	.	Specifications for the Construction and Reconstruction, as well as the

Test Methods for the Packages and Packaging for Hazardous Substances, Materials and Wastes.

NOM-052-ECOL/1993	Establishes the Characteristics of the Hazardous Wastes, a List of them and the Limits that make a Waste Hazardous due to its Toxicity  to the Environment.

  - Definitions

Package.- Any utensil or wrapping in which the product is contained, for its distribution or    sale.

Packaging.-  A material that wraps, contains and protects adequately the pre-packed products, that facilitates and resists the storage and transportations operations.

Drum.-   Is a cylindrical package and packaging with flat or convex surfaces, made of metal, fiberboard, plastic, plywood or any other appropriate material. This definition also includes the packages and packaging that have other shapes, for instance, the round ones with conical neck packages and packaging or the packages and packaging with a cubic shape.

Jerrican.- Is a metal or plastic package or packaging of rectangular or polygonal cross-section.

Wooden barrels,- Are packaging made of natural wood, of round cross-section, having convex walls, consisting of staves and heads and fitted with hoops.

Boxes.- Are packaging with complete rectangular or polygonal faces, made of metal, wood, plywood, reconstituted wood, fiberboard, plastics or other suitable material. Small holes for purposes such as ease of handling or opening, or to meet classification requirements, are permitted as long as they do not compromise the integrity of the packaging during transportation

Receptacles.-   Are containment vessels for receiving and holding substances or articles, including any means of closing.

NOM-007-SCT2/2002     Page   5

Bags or Sacks.-  Are flexible packaging made of paper, plastics film, textiles, woven material or other suitable materials.

Crates.-  Are outer packaging with incomplete surfaces and open spaces between its constructive elements.

Composite packaging.-   Are packaging consisting of an outer packaging and an inner receptacle so constructed that the inner receptacle and the outer packaging form an integral packaging. Once assembled it remains thereafter an integrated single unit; it  is filled, stored, transported and emptied as such.

Packaging.-			Includes a metal drum that are:

Reconditioned

a)   Has been cleaned to original materials of construction, with all former contents, internal and external corrosion, and external coatings and labels removed;

- Restored to original shape and contour, with chimes (if any) straightened and sealed and all non-integral gaskets replaced; and

- Inspected after cleaning but before painting, with rejection of packaging with visible pitting, significant reduction in material thickness, metal fatigue, damaged threads or closures, or other significant defects.

  - Specification for the Marking of Packages and Packaging

  - Every package and packaging destined to be utilized for the transport of  hazardous materials, substances or wastes , must bear perfectly visible markings, indelible, legible, and its size will be in proportion with the package and packaging.

Besides, on each package and packaging the transport official designation of the hazardous material, substance or waste, must appear (appropriate name of the shipping), with the corresponding number of identification from the United Nations, preceded by the letters “UN”. In the case of substances of division 1.4, compatibility group “S”, also indicate the division and the letter of the compatibility group, unless they go marked with the label 1.4 S”. Example of marking:

Liquids, corrosive, N.O.S. (Caprilil Chloride) UN 1760

For packages and packaging with a gross weight of more than 30 kg., the marking or a duplicate of it must appear on the upper part or at a side of the package or packaging.

Letters, numbers and symbols must be at least 12 mm. high, except for packages and packaging of less than 30 liters of capacity in which case they must be at least of 6 mm. high and for packages and packaging of 5 liters or les than 5 kg., they should be of an appropriated size.

The marking should indicate:

- The United Nations symbol for package and packaging.

u

n

NOM-007-SCT2/2002    Page   6

This symbol must not be utilized except to certify that a determined package or packaging, complies with the specifications of the Mexican Official Standard NOM-024-SCT2, Specifications for the Construction and Reconstruction, as well as the Test Methods of the Packages and Packaging of the Hazardous Substances, Materials and Wastes, and with is established in the Regulation for the Land Transport of Hazardous Materials and Wastes.

Only the packages and packaging that have passed successfully the performance test criteria prescribed on the Mexican Official Standard NOM-024-SCT2, must be marked with the United Nations symbol.

In the case of the packages and packaging of metal manufacture with embossed or engraved marks, upper case letters can be used for the “UN” symbol.

b)  The code for the designation of the type of package or packaging consists of:

bi)  An arabic digit that indicates the type of package or packaging, for example, (drum, jerrican, etc.) followed by:

bii)  One or several upper case letters in latin characters that indicate the nature of the material, for example, (steel, wood, etc.) followed, when necessary, by:

biii)  An Arabic digit that indicates the category of the package and packaging within the type to which it belongs to.

In the case of the composite packages and packaging,  on the second place of the code, there must be two upper case letters, in latin characters. The first one indicates the material of the inner package and the second the one for the outside package and packaging.

In the case of the combined packages and packaging and those for infectious substances, it should only be used the code corresponding to the outside package and packaging.

The code of the type of package or packaging may be followed by the letters “T”, “V”, or “W”.

    - Letter “T” indicates an emergency package o packaging that complies with the provision on     paragraph 7.1.10 of the Mexican Official Standard NOM_024-SCT2.

    - Letter “V” indicates an special package or packaging that complies with the provisions of paragraph 7.1.6.1 of the Mexican Official Standard NOM-024-SCT2

    - Letter  “W”, indicates that the package or packaging, although it is of the type designed by the code, it has been manufactured according to different specifications to those indicated on  section 6 of the Mexican Official Standard NOM-024-SCT2 and it is considered equivalent to what it is prescribed on paragraph 5.18 of the Mexican Official Standard NOM-024-SCT2.

For the designation of numerals and letters referred to on previous paragraphs, whatever is indicated on Table 1, will remain.

- A code made of two parts.

- A letter that indicates that the packaging group or groups for which the model have passed the tests:

X,	For packaging groups I. II, III;

Y,	For packaging groups II and III;

Z,	For packaging group III only.

- In the case of packages and packaging that does not have an inner package destined to the transport of liquids, the relative density, rounded to the first decimal, (This indication can be omitted if the relative density does not exceed 1.2);  in the case of packages and packaging destined to solid substances or of inner package or packaging, the maximum gross weight must be shown in kilograms.

d)                  Either the letter “S”, that indicates that the package or packaging is destined to the transport of solids or of inner packages and packaging, or else in the in the packages and packaging (other than the combination ones), destined to the transport of liquids and that they passed the hydrostatic test, meaning the test pressure in kilopascals (k Pa), rounded to the nearest tenth.

NOM-007-SCT2/2002     Page  7

      - It must also include the last two digits of the year of manufacture of the package or packaging. the types with assigned codes “1H” and “3H” must also have the month of manufacture; this can be marked on a different place than the other markings. To such end, the appropriate end is the following:

      - The distinctive sign of the country of manufacture. In the case of  packages and packaging manufactured in Mexico the letters MEX must be placed on them.

      - The package and packaging must also have the name or another mark that identifies the manufacturer or the Test Laboratory prescribed by the agency, that certifies that the packages and packaging satisfy the construction and reconstruction specifications and the test methods for packages and packaging of hazardous substances, materials and wastes pointed out on NOM-024-SCT.

  - Besides the indelible marks that appear on point 5.1, each barrel or drum of new metal of a capacity larger than 100 litters must carry the marks described on point 5.1, sections a) to e), in the lower side, with an indication of the nominal thickness of at least the metal used in the body (in mm. up to 0.1 mm.), This marking must be made on a permanent manner (embossed example). When the nominal thickness of the top and bottom of a metal drum is thinner than the body, the nominal thickness of the upper part, the body and the down portion must be marked on a permanent manner. For example: “1.0-1.2-1.0” or “0.9-1.0-1.0”.

The theoretical metal thicknesses will be determined according to the appropriate steel standards or its equivalents. The markings indicated on sections f) and g) of paragraph 5.1, will not be applied in permanent form (embossed).

    - For barrels or drums manufactured of reconditioned metals, if there are no changes of the type of package or packaging and without replacement or elimination of integrated structural components, required markings does not have to be permanent. All other drums  of reconditioned metals will carry the markings as indicated in sections from a) to e) of point 5.1 in a permanent manner (embossed) in the upper part or in the body.

    - Metal drums made of materials designed to be repeatedly reconditioned, (example: stainless steel),  may carry the markings indicated on point 5.1, sections f( and g) in a permanent manner.

  - Every reusable package or packaging, that can be subjected to a reconditioning process that may erase the markings, must carry the markings indicated on sections from a) to e) of point 5.1, in the same order indicated on in a permanent manner, (embossed or in relief). Permanent markings are those which are considered that can resist the reconditioning process. In the case of packages or packaging that are not metal drums with a capacity over 100 litters, these permanent marks may replace the permanent marks mentioned on point 5.1.

  - In the case of reconditioned packages and packaging, indelible marks must be included, in the appropriate order according to what it is indicated :

      - The distinctive sign of the country in which the reconditioning has been performed, for which the distinctive symbol used by automobiles in the international traffic (MEX)

      - Name and authorized symbol of the re-manufacturer.

      - Year of the reconditioning, the letter “R” that indicates reconditioning and the letter ”L” in case that the packages and packaging have passed the hermetic test  referred to on the Mexican Official Standard NOM-024-SCT2, Specifications for the construction and reconstruction  as well as the Test Methods of the Packages and Packaging of the Hazardous Materials and Wastes.

These marks must be placed by the referred one on point 5.1 and may substitute the markings established on sections f) and g).

  - When after reconditioning, the markings required on point 5.1, sections a) to d) do not appear on the top part or on the sides of a metal drum, the re-manufacturer  must apply the indications of point 5.4, sections h), i) and j). These markings must not identify a larger capacity vessel for which the original design type has been tested and marked.

NOM-007-SCT2/2002     Page 8

Following it is shown an example of markings for new and reconditioned packages and packaging.

NEW

4G/Y145/S/83/

MEX/VL 824

_________________________________________________________________________________

CHARACTERISTIC			NUMBER OR LETTER			ORIGIN

Box							4			Table 1

Fiberboard						G			Table 1

Packing Group II and III				Y			Section c(I)

Gross weight of material				145			Section c (II)

Solid material						S			Section d

Year of manufacture					83			Section f

Country of manufacture				Mex			Section f

Manufacturer’s mark					VL 824			Section g

RECONDITIONED

1A1/Y 1.4/150/83/

MEX/VL 824 RL				              		CHARACTERISTIC		NUMBER OR LETTER			ORIGIN

Drum							1			Table 1

Steel							A			Table 1

Fixed top						1			Table 1

Packing Group II,III					Y			Section c (I)		 Relative density					1.4			Section c (II)

Hydrostatic test pressure				150			Section d

Year of manufacture					83			Section e

Country of manufacture				Mex			Section h

Manufacturer’s mark					VL 824			Section I

Reconditioned					R			Section j

Hermetic Test						L			Section j

The following numbers show the type of package and packaging:

        - Drum

        - Barrel (Wood drum)

        - Jerrican

        - Box

        - Sack

        - Composite package

        - Pressure vessel

The following upper case letters indicate the type of material:

A	Steel

B	Aluminum

C	Natural wood

D	Plywood

F	Reconstituted wood

G	Fiberboard

H	Plastic

L	Fabric

M	Multiwall paper

N	Metal (except steel and aluminum)

P	Glass, porcelain or stoneware

NOM-007-SCT2/2002    Page 9

TABLE 1

CODES ASSIGNED TO DIFFERENT TYPES OF PACKAGES AND PACKING

_________________________________________________________________________________

TYPE OF PACKAGE      MATERIAL	    CATEGORY		                DESIGNATION CODE

AND PACKAGING

1. Drums		  A. Steel 1	      Fixed top			     1A1

_____________      Removable top		     1A2

B. Aluminum      1. Fixed top			     1B1

2. Removable top		     1B2

D. Plywood					     1D

G. Fiberboard					     1G				                 	               H. Plastic	     1. Fixed top			     3H1

______________________________  2. Removable top		     3H2

2. Barrels, Casks	C. Wood	     1. With bung			     2C1

2. Removable top		     2C2

3. Jerricans		A. Steel		     1. Fixed top			     3A1

_____________  2. Removable top		     3A2

B. Aluminum	     1. Fixed top			     3B1				            	            	     _           2. Removable top		     3B2

H. Plastic	     1. Fixed top			     3H1

______________________________  2. Removable top		     3H2

4. Boxes		A. Steel						     4A

B. Aluminum					     4B

C. Natural wood    1. Ordinary			     4C1

______________ 2. With siftproof walls		      4C2

D. Plywood					      4D

F. Reconstituted wood				      4F

G. Fiberboard					      4G

H. Plastic	       1. Expanded		      4H1

2. Rigid			      4H2

5. Sacks/Bags	H. Plastics material 1. No inside lining		      5H1

2. Non sieving		      5H2

3. Water resistant		      5H3

H. Plastic film					      5H4

L. Textiles	         1. No inner liner		      5L1

2. Sift-proof		      5L2

_______________  3. Water resistant		      5L3

M. Paper	         1. Multi wall		      5M1

2.. Multi wall, water resistant  5M2

6. Composite 		H. Plastic 	         1. In steel drum		      6HA1

packaging	 	receptacle	         2. In steel crate or box	      6HA2

1. In aluminum drum	      6HB1

2. In aluminum crate or box    6HB2

In wooden box		      6HC

1. In plywood drum	      6HD1

2. In plywood box		      6HD2

1. In fiber drum		      6HG1

2. In fiberboard box	      6HG2

1. In plastics drum		      6HH1

____________ 2. In solid plastics box	      6HH2

NOM-007-SCT2/2002    Page 10

P. Glass, porcelain  1. In steel drum		     	 6PA1

or stoneware	         2. In steel crate or box	     	 6PA2

receptacle	         1. In aluminum drum	     	 6PB1

2. In aluminum crate or box   	 6BP2

In wooden box		      	 6PC

1. In plywood drum	     	 6PD1

2. In wickerwork hamper	     	 6PD2

1. In fiber drum		      	6PG1								         2. In fiberboard box	     	 6PG2                       						         1. In expanded plastics packing	 6PH1							                       2. In solid plastics packaging	 6PH2

_________________________________________________________________________________

- Bibliography

Recommendations on the Transport of Dangerous Goods, issued by  the United Nations Organization, Eleventh Edition, New York and Geneva, 1999; Recommendations on the Transport of Dangerous Goods, Model Regulation, issued by the United Nations Organization, Eleventh  edition, New York and Geneva, 1999.

Federal Law on Metrology and Standardization, and its Regulation.

ISO Standard 3574:1984 for Steel.

7.     Harmonization with international standards and regulations

This Mexican Official Standard is equivalent to the Recommendations on the Transport of Dangerous Goods, Model Regulations, Eleventh revised edition, of the United Nations  Chapter 6.1, points 6.1.1, 6.1.2, 6.1.3 and 6.1.4

  - Compliance

In conformity with the Regulations for the Transport of Hazardous Materials and Wastes, compliance of this Mexican Official Standard is obligatory.

  - Inspection

The Secretariat of Communications and Transport, through the competent directorates generals is the authority to supervise  compliance of this Mexican Official Standard.

  - Evaluation of Conformity

The Evaluation of Conformity will be performed under the scheme set forth by the Federal Law on Metrology and Standardization, by the Organisms of Certification and the accredited and approved Test Laboratories or else by the Secretariat.

10.1.- Every package or packaging destined to contain and transport hazardous substances and wastes, must show, stamped, a code that includes the UN marks that guarantees having been manufactured according to what is established on the Mexican Official Standard NOM-024-SCT2, including the code sequence as it is described in this Standard.

10.2.- The Test Laboratories will be the responsible entities for performing the integrity tests as required by Standard NOM-024-SCT2. Assigning the corresponding UN marking, in coordination with this agency as it is defined in this Standard and to provide the test reports that will include the obtained results.

NOM-007-SCT2/2002     Page 11

10.3.-The Product Certification Organisms will be the ones responsible of granting the certifications to the manufacturer or remanufacturer, when solicited by them, recognizing the Test Laboratories.

10.4.- Every manufacturer or remanufacturer of packages and packaging, in order to stamp the UN code must have the favorable report from the accredited Testing Laboratory. These Laboratories will coordinate the codes assignment with the Secretariat.

10.5.- The period in which the documents granted to the manufacturer by the Test Laboratories, or the Secretariat, are in force, will be of two years, subject to a yearly inspection.

10.6.- For the application to obtain the Accreditation and Approval, the Certification Organisms and Test Laboratories, will observe the Convocation that for that effect will be issued by the Secretariat, duly complying with the requisites established therein, demonstrate their capacity and technical infrastructure, trained personnel and knowledge on the Standardization, besides the specific aspects established within the same Convocation, in compliance with what is established in the Federal Law on Metrology and Standardization and its Regulation.

11.- Sanctions

The non-compliance of the provisions contained in this Mexican Official Standard will be sanctioned by this Secretariat, according to what is established in the Regulation for the Transport of Hazardous Materials and Wastes and other legal ordainments that may result applicable , without interfering with other sanctions applied by other Federal Agencies, while exercising their authority or the civil or penal responsibility resulting from it.

12.- Validity

This Mexican Official Standard will be in force 90 days after its publication in the Federation Official Journal.

13.- Transitory

FIRST.- As this Mexican Official Standard enters in force, will substitute Mexican Official Standard NOM-007-SCT2/1994 published in the Federation Official Journal on August 18, 1995.

SECOND.- The Secretariat of Communications and Transport will request from the Secretariat of Economy, in compliance with the terms and requisites established by the Federal Law on Metrology and Standardization and its Regulation, to carry out the necessary formalities in order to harmonize agreements for mutual recognition of the results of the evaluation of conformity, as well as, of the accreditations granted with foreign and international official institutions, in a period not to exceed two years as of the start of this Mexican Official Standard being in force .

Also, the Secretariat of Communications and Transport will promote that the entities of accreditation and the accredited persons also reach agreements of similar nature to those referred to on the previous paragraph.